Supplementary Information

TITLE PAGE

Title:

Hydrosheaf: An Integrated Validation Framework for Coupled Groundwater Residence-Time, Flow-Topology and Inverse Hydrogeochemical Modelling in Data-Limited Aquifers

Authors:

Dickson Abdul-Wahaba*, Ebenezer Aquisman Asareb,  Dickson Adomakoa, Gibrilla Abassc, Samuel Ganyagloa,d,

Affiliations:

a Department of Nuclear Science and Applications, School of Nuclear and Allied Sciences, University of Ghana, Atomic-Kwabenya, Accra, Ghana

bNuclear Chemistry and Environmental Research Centre, National Nuclear Research Institute (NNRI), Ghana Atomic Energy Commission (GAEC), Box LG 80, Legon-Accra, Ghana

cDepartment of Water Resources and Aquaculture Management, School of Sustainable Development, University of Environment and Sustainable Development, Somanya, Ghana

dWater Resources Research Centre, National Nuclear Research Institute, GAEC, Box LG 80, Kwabenya-Accra, Ghana

*Corresponding Authors:

Dickson Abdul-Wahab;

Email: dabdul_wahab11@yahoo.com

ORCiD: https://orcid.org/0000-0001-7446-5909

Supplementary Table

Table S1

| Data type | Required field | Unit | Required/optional | QC rule | Used in module |
| --- | --- | --- | --- | --- | --- |
| Location | latitude/longitude or x/y | decimal degrees or projected | required | valid coordinate range | graph |
| Elevation/head | elevation, head_m | m | recommended | non-missing preferred | topology |
| Major ions | Ca, Mg, Na, K, HCO3, Cl, SO4, NO3 | mmol/L or mg/L | required | charge-balance screening | chemistry |
| Conditional ions | F, Fe, PO4, Br | mg/L or ug/L | optional | detection-limit handling | logic gates |
| Stable isotopes | d18O, d2H | permil | optional/enhanced | tracer range check | transport |
| Nuclear tracers | tritium, 14C | TU, pmC | optional | tracer range check | age |
| Field parameters | pH, EC, temperature, DO/Eh | field units | recommended | field range check | geochemistry |

Table S2

| Process family | Reaction/process term | Chemical signature | PHREEQC/SI rule | Logic gate | Allowed direction |
| --- | --- | --- | --- | --- | --- |
| carbonate dissolution | calcite | Ca, HCO3 | SI/forward check when available | reaction dictionary | nonnegative |
| carbonate dissolution | dolomite | Ca, HCO3, Mg | SI/forward check when available | reaction dictionary | nonnegative |
| evaporite dissolution | gypsum | Ca, SO4 | SI/forward check when available | reaction dictionary | nonnegative |
| evaporite dissolution | halite | Cl, Na | SI/forward check when available | reaction dictionary | nonnegative |
| silicate weathering | albite | HCO3, Na | SI/forward check when available | reaction dictionary | nonnegative |
| redox process | pyrite_oxidation_aerobic | Fe, SO4 | SI/forward check when available | reaction dictionary | nonnegative |
| nitrate input | NO3src | NO3 | SI/forward check when available | reaction dictionary | nonnegative |
| redox process | denit | HCO3, NO3 | SI/forward check when available | reaction dictionary | nonnegative |
| ion exchange | CaNa_exch | Ca, Na | SI/forward check when available | reaction dictionary | signed |
| ion exchange | NaCa_exch | Ca, Na | SI/forward check when available | reaction dictionary | nonnegative |
| ion exchange | MgNa_exch | Mg, Na | SI/forward check when available | reaction dictionary | signed |
| ion exchange | NaMg_exch | Mg, Na | SI/forward check when available | reaction dictionary | nonnegative |

Table S3

| Parameter | Value/range | Used in module | Scientific rationale |
| --- | --- | --- | --- |
| n_realisations | 100 | benchmark | Monte Carlo recovery ensemble |
| lambda_l1 (default) | 0.002 | sparse inversion | default sparse-recovery penalty |
| AICc-selected lambda | 0.0483 | site model selection | selected for Lower Anayari and Talensi only |
| major_ion_rel_sigma | 0.04 | synthetic data | analytical uncertainty on chemistry |
| isotope_sigma_permil | 0.50 | isotope validation | stable-isotope measurement noise |
| transport_models_enabled | evap, mix | edge fitting | minimum transport alternatives for M2 |
| PSI trials | 30 default | field uncertainty | edge stability under perturbation |
| SI threshold | 0.2 diagnostic band | forward checks | thermodynamic feasibility screening |

Table S4

| Site | Sample ID | Latitude | Longitude | Elevation (m) | Aquifer setting | Land-use/mining influence | Available tracers | Completeness score |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Talensi | TGW1 | 10.71 | 0.82 | 217 | Crystalline basement | Mining/Agri | stable isotopes; no nuclear tracers | 1.00 |
| Talensi | TGW2 | 10.71 | 0.81 | 222 | Crystalline basement | Mining/Agri | stable isotopes; no nuclear tracers | 1.00 |
| Lower Anayari | PUB1 | 10.91 | -1.06 | 217 | Alluvial/Basement | Agriculture | stable isotopes; no nuclear tracers | 1.00 |
| Lower Anayari | PUB2 | 10.91 | -1.06 | 217 | Alluvial/Basement | Agriculture | stable isotopes; no nuclear tracers | 1.00 |

Table S5

| Edge ID | From node | To node | Distance (km) | Elevation/head relation | Edge confidence | Age consistency | Chemical match R2 | Dominant reaction | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Talensi_48_61 | TGW49 | THDW10 | 2.52 | downgradient | 1.00 | field tracer absent | 1.00 | carbonate dissolution | screening-level demonstration |
| Talensi_45_44 | TGW46 | TGW45 | 1.93 | upgradient/flat | 1.00 | field tracer absent | 1.00 | nitrate input | screening-level demonstration |
| Talensi_48_42 | TGW49 | TGW43 | 2.54 | downgradient | 1.00 | field tracer absent | 1.00 | redox process | screening-level demonstration |
| Manu_30_28 | GOB38 | GOB36 | 1.31 | downgradient | 1.00 | field tracer absent | 0.99 | evaporite dissolution | screening-level demonstration |
| Talensi_51_26 | TGW52 | TGW27 | 3.31 | upgradient/flat | 1.00 | field tracer absent | 0.99 | nitrate input | screening-level demonstration |
| Talensi_28_27 | TGW29 | TGW28 | 1.57 | downgradient | 1.00 | field tracer absent | 0.99 | nitrate input | screening-level demonstration |

Supplementary Figures

Supplementary Figure S1. Public USGS residence-time parity check using the M3 age-fraction-constrained benchmark. Among 1,272 rows, 1,249 had finite log10 errors; R² = 0.71, median absolute log10 error = 0.17, 61.4% were within a factor of two and 85.9% within a factor of ten. The USGS ages and age fractions are reported model outputs, so this is screening-level parity rather than direct true-age validation or full TracerLPM equivalence.

Supplementary Figure S2. PHREEQC-compatible forward-feasibility proxy. Panel (a) shows the RMSE distribution (median = 0.062 mmol/L); panel (b) relates RMSE to Nash–Sutcliffe efficiency (median NSE = 0.999); and panel (c) shows percent bias (median PBIAS = 0.9%). The simulator uses locked PHREEQC-consistent saturation fields; live PHREEQC kinetic execution remains pending.

Supplementary Figure S3. Field-scale chemical discovery fit for the Lower Anayari and Talensi groundwater networks. The histograms show edge-level chemistry R² for generated coordinate/head-proxy graphs: median R² = 0.60 for Lower Anayari and 0.77 for Talensi. These fits describe hydrochemical closure of a field demonstration and do not provide independent process-truth validation.